# HIPO DCS Software Interface

#### Document Number HI-Z10-11

Authors:	Ted Dunham, Peter Collins, Georgi Mandushev, and Saeid Zoonematkermani

Date: 	30 August 2013, Version 2.0

HIPO will be operated using the Lowell Observatory Instrumentation System (LOIS) control software. This document describes the proposed FITS keyword list for LOIS as used on SOFIA. The document also includes a description of the HIPO camera modes, as well as a proposed list of searchable FITS keywords.

Table of Contents

I.	FITS Keyword List and Definitions	2

1)	FITS Standard Keywords	2

2)	LOIS Keywords	4

3)	Observatory Keywords	4

4)	Observation Description Keywords	5

5)	Date and Time Keywords	6

6)	Environmental Keywords	6

7)	Camera Keywords	7

8)	Telescope Keywords	9

9)	Annotation Keywords	10

10)	Instrument Keywords	11

11)	Data Collection Keywords	12

12)	DCS Pipeline Related Keywords	12

13)	Mission Management Keywords	12

14)	Chopping Keywords	13

15)	Nodding Keywords	13

16)	Dithering Keywords	13

17)	Mapping Keywords	13

18)	Scanning Keywords	14

II.	HIPO Camera Modes	14

1)	Find	15

2)	Single Frames	16

3)	Series	16

4)	Basic Occultation	17

5)	Fast Occultation	17

6)	Pipelined Occultation	18

7)	Fast Dots	19

8)	Slow Dots	20

9)	Strips	21

III.	List of Searchable FITS Keywords	22

IV.	Appendices	22

1)	FITS file naming convention	22

2)	Items and questions still to be worked out	22

V.	Figures	23

# FITS Keyword List and Definitions

The FITS keywords to be used by LOIS for HIPO (High-speed Imaging Photometer for Occultation) are described in this section. They are grouped into logical subsections as listed in the Table of Contents. Within each group the FITS keywords are separated into the following categories:

- LOIS Standard: keywords that will go into any FITS header written by the LOIS software package

- SOFIA Required: keywords that are required for the SOFIA and DCS headers

- SOFIA Optional: keywords that are optional for SOFIA

In this document the word “pixel” means the physical detector pixel. The word “data element” is used to denote the elements of the FITS data array, the numbers that are actually written to and read from the FITS file. These are distinct if the CCD readout is binned.

The HIPO header is currently built from the following data streams:

- basic camera and file information provided by C language code in the camera, instrument and storage modules of LOIS. Much of this will come from configuration files. In a few cases modules will access TCL variables.

- MCCS housekeeping telemetry passed semi-transparently from the JAVA MCS thread 1Hz subscription, as selected by the telemetry subscription configuration file MCS_Subscriptions.xml.

and in addition,

- Process 1 telemetry which is data taken from the Housekeeping and passed into structures and provided as keywords by the deeper level C code in the modules of LOIS. In some cases, like SOFIA focus, the same information is available as both a direct telemetry and Process 1 keyword. In some cases, for example, the WCS keywords, the processing involves multiple structure items based on telemetry.

- Constant telemetry keywords, which are single-valued keywords generated from MCS_Subscriptions.xml with no associated HK tag.

## FITS Standard Keywords

FITS standard keywords are defined as all keywords used for defining the data and image size.

a) LOIS Standard

SIMPLE	Boolean	/	If T (true), the FITS file conforms to the FITS standard. T for HIPO.

BITPIX	Integer	/	Number of bits per data element. Valid values are

8: 	character or unsigned integer

16: 	16-bit signed integer (HIPO uses this case)

32: 	32-bit signed integer (long integer)

-32: 	32-bit floating point single precision

-64:	64-bit floating point double precision

NAXIS	Integer	/	Number of axes in the data array. These could be just X and Y (columns

and rows), in which case NAXIS = 2, or X, Y and time (NAXIS = 3).

NAXISn	Integer	/	Number of data elements along data axis n, n = 1,NAXIS. Each data

element is associated with an index running from 1 to NAXISn along the

corresponding axis. Suppose that the FITS file contains 128x256 data

elements. Then NAXIS1 = 128, NAXIS2 = 256, and each element will

be referenced by two indices running from 1 to 128, and from 1 to 256,

respectively. The indices simply show the position of each data element in

the data file.

The next two keywords (BZERO and BSCALE) are used when the data array values are not the true physical values. The keywords describe a linear transformation equation between raw data values and the true values: TRUE_VALUE = BZERO + BSCALE x ARRAY_VALUE. HIPO uses these to map the ADC’s unsigned 16-bit integer output to a signed 16-bit integer to comply with the FITS standard.

BZERO  	Float	/	The zero point in the scaling equation

BSCALE 	Float	/	The coefficient of the linear term in the scaling

BUNIT  	String	/	Data units (ADU, electrons, etc.) after application of BSCALE and BZERO

The next four keywords (CTYPEnU, CRPIXnU, CRVALnU and CDn_m) are used to describe the coordinate system of the data in the FITS file. Each data element has one or more indices, each index runs from 1 to NAXISn. On the other hand, each axis represents a physical coordinate (X or Y position, RA, DEC, time, etc.). The C-keywords describe the relations between the data index along each axis and the physical coordinate represented by that axis.

CTYPEnU	String	/	Name of the coordinate represented by axis n (X, Y, RA, DEC, time, etc.)

For HIPO this is linear unbinned CCD pixels along axes 1 and 2, and UT

seconds relative to UTCSTART for axis 3.

CRPIXnU	Float	/	The location of a reference point along axis n, in units of the data index

along NAXISn. CRPIX1U differs from 1 in the HIPO headers only if there are prescan pixels (CRPIX1U = 1 + total number of binned prescan pixels; see Figure 2). This is rounded if the number of binned prescan pixels is fractional. CRPIX2U and CRPIX3U are always 1 in the HIPO headers.

CRVALnU	Float	/	The value of the coordinate CTYPEnU at reference point CRPIXnU. For

example, if CTYPE1U is wavelength, then CRVAL1U = 8070.0 means that

the wavelength at data element 25 (the value of CRPIX1U) is 8070.0. In 				the HIPO headers, CRVAL1U and CRVAL2U represent the location of the

first physical pixel in unbinned pixel coordinates in the physical imaging

area of the CCD. By this definition the 16 dark reference columns (see Figure 3) are included in the physical image area. CRVAL3 is discussed in the text at the end of this section.

CDn_mU	Float	/	The WCS matrix coefficient relating the data index along axis n to the

physical image pixels along axis m. Please see below for an illustration of

the data-to-imager WCS matrix.

CFINTn	Float	/	The data fill factor. Those have different meaning depending on NAXISn.

CFINT1 and CFINT2 are the pixel fill factors in column and row direction,

respectively. Those are set to one for HIPO. CFINT3 is the duty cycle of

the observing mode, CFINT3 = EXPTIME/CRDELT3.

The WCS matrix relating the data array elements to the physical imager pixels:

Let X and Y denote the imager pixel indices along X and Y, and XD and YD denote the data indices along the two axes. Then the values of X and Y can be calculated from the known values of XD and YD using this matrix equation:

CD1_1U  CD1_2UCD1_2U  CD2_2UXD-CRPIX1UYD-CRPIX2U + CRVAL1UCRVAL2U = XY

This is the first WCS matrix, which is used to map the data elements to the imager pixels. There is a second WCS matrix (described later), which maps the data elements to the sky.

The time axis of the HIPO data files deserves further elaboration. There are two fundamentally different readout timing approaches: software triggering and hardware triggering. Hardware triggering will be used when precise timing is required. Software triggering can be used as a fallback mode, and is sometimes useful in test environments where hardware triggering is not possible.

The HIPO headers have CRPIX3U set to 1. CD3_3U is the period of the integration and readout activity for the mode in question as described immediately below. CRVAL3U is calculated such that the midtime of the exposures in the data file may be found as UTCSTART + (n-1)*CD3_3U + CRVAL3U where n is the one-based integration number. There is one exception to this calculation for the first integration in a hardware triggered time series that is described below. The values of CD3_1U and CD3_2U will always be zero.

Hardware Triggering:

In the hardware triggering case a triggering waveform is supplied to the timing board of the CCD controller, and DSP code on the timing board uses the edges of the trigger waveform to coordinate the various phases of the CCD readout. The waveform is high prior to initiation of the data acquisition process, and drops on the exact UT second corresponding to UTCSTART when a hardware triggered exposure begins. The trigger is low for a time equal to the trigger period minus one count of the trigger timebase and high for one count of the trigger timebase. There are three selectable timebase frequencies available: 1 Hz, 1 KHz, and 1 MHz. The falling edges of the trigger waveform are the critical ones; where the rising edge occurs is not relevant.

The CCD flushes during the period prior to the first falling trigger edge, then begins to integrate. On successive falling edges, the mode-dependent readout process (see the pseudo-code found later in this document) is initiated. Integration resumes for the following integration once the part of the readout that affects integration is complete. CD3_3U is equal to the trigger period in this case and EXPTIME is the well-defined CD3_3U period minus the readout overhead. In hardware triggering CRVAL3U is set to CD3_3U – (EXPTIME/2). The readout overhead is a function of many readout parameters and may need to be carefully calculated and/or measured if it is critical. For a given configuration it is rigorously reproducible and stable to the level of a tight DSP loop, about 250 ns.

It should be noted that the hardware triggering just described causes the first integration to have an exposure time equal to CD3_3U, whereas all subsequent integrations have exposure time equal to EXPTIME. The affects the signal level in the first integration relative to the following ones, and also the midtime of the first integration calculated according to the method described above will be incorrect by half the readout overhead time. This is generally only of academic interest, but is pointed out for completeness.

Software Triggering:

In software triggering the observer defines the exposure time rather than the integration interval and the exposure timing capability in the CCD controller is used to make the exposure time equal to EXPTIME within the capability of the CCD controller’s hardware. The interval between exposures, CRDELT3, is then the well-defined EXPTIME plus the readout overhead. In software triggering CRVAL3 is set to EXPTIME/2. If software triggering is used for a critical timing application it is essential to measure the actual CRDELT3 value by external means since the calculation in LOIS is not guaranteed to be sufficiently precise for critical timing measurements. HIPO includes 1 Hz blinking calibration LEDs for this purpose.

## LOIS Keywords

LOIS keywords are used to define the version and the types of software modules that were used to create the FITS file.

a) LOIS Standard

LOISVERS	String	/	LOIS Version

LCAMMOD 	String	/	LOIS camera module

LTELMOD 	String	/	LOIS telescope module

LINSTMOD 	String	/	LOIS instrument module

The following version strings include a part indicating the software revision level and an optional second part that is included if the same code has been customized for a specific function. In the timing board code, for example, this can be used to indicate different readout speed values or parallel clock speeds. Examples of this string are V1.3/1, V1.7/7/2, and V1.4/2_a.

PCIDSPV	String	/	DSP code version ID for PCI interface board for CCD controller

UTILDSPV	String	/	DSP code version ID for utility board in the CCD controller

TIMDSPV	String	/	DSP code version ID for timing board in the CCD controller

## Observatory Keywords

Observatory keywords are used to describe where and by whom the FITS file was created.

a) LOIS Standard

OBSERVER	String	/	Observer(s)

OBSAFFIL 	String	/	Observer(s) affiliation

OBSERVAT 	String	/	Observatory name (e.g., SOFIA)

HGPS_LON	Float	/	Longitude of HIPO GPS unit for the corresponding channel at UTCSTART,

measured from the Prime Meridian with positive values going East.

HGPS_LAT	Float	/	Latitude of HIPO GPS unit for the corresponding channel at UTCSTART.

HGPS_ALT	Float	/	Altitude of HIPO GPS unit in meters for the corresponding channel at

UTCSTART.

DATUM	String	/	The definition of longitude and latitude. Set to ‘WGS84’ (World Geodetic

System 1984) in the HIPO headers.

CREATOR	String	/	File creation task (e.g., LOIS)

FILENAME	String	/	Original camera disk file name. See Appendix 1 for the HIPO convention.

ORIGIN	String	/	Origin of the FITS file (e.g., ‘HIPO – Lowell Observatory’).

b) SOFIA Required

OPERATOR	String	/	Telescope operator

c) SOFIA Optional

NONE

## Observation Description Keywords

Observation keywords describes the observation that was initiated to create the FITS file.

a) LOIS Standard

DATASRC	String	/	Data source defined in DCS_SI_01: ASTRO or LAB for object frames, 
			CALIBRATION for flat, dark, and bias frames.

OBSTYPE	String	/	Observation type: One of: OBJECT, FLAT, DARK, BIAS.

IMAGEID	Integer	/	Image identification number of the FITS extension (always 1 for HIPO)

OBSERNO	Integer	/	Sequential 4 digit observation number for the observing session

OBJNAME	String	/	At this time HIPO makes this value the same as OBJECT

OBJECT	String	/	Object title (determined by the observer), e.g., ‘NGC6752 West’

b) SOFIA Required

OBSSTAT	String	/	Observation status. Either OK or ERROR. Currently OBSSTAT is set to

ERROR only if a gross error in triggering occurs (i.e., many triggers

missed). If other errors occurs, such as a readout error from a timeout

resulting in a spontaneous abort, or if data acquisition was aborted, the FITS files are not saved.

SRCTYPE	String	/	Source type (e.g., ‘POINT SOURCE’).

KWDICT	String	/	Keyword Dictionary Version. Currently set to ‘DCS_SI_01_E’, where the

final letter is the document revision for the SOFIA DCS ICD or keyword

document.

OBS_ID	String	/	SOFIA Observation Identification. Numeric fields contain leading zeroes.
			The string format is :
				yymmdd_MMM_LLIInnnn_ss[_x] 
			with fields defined as follows:
				yymmdd is a set of three 2-digit fields for UT year, month, day 
				MMM and LL are the flight (FLIGHTNO) and leg (FLIGHTLG) numbers
				II is the instrument ID, HB for HIPO Blue and HR for HIPO Red
				nnnn is the 4-digit observation number (OBSERNO) with leading zeroes
				ss is the subframe number (SUBARSER), zero if full frame, and
				_x is conditionally present if file overwriting prevention resulted in a file
				with a prepended single letter (see Appendix 1). The ‘x’ is the 
				prepended single letter from the filename.

AOR_ID	String	/	Observation request identifier. Currently it matches OBS_ID.

PLANID	String	/	Observing plan identification.

DB_OBSID	Integer	/	Observation ID for archiving purposes.

FILEGPID	String	/	File Group Identifier, set by the observer.

c) SOFIA Optional

NONE

## Date and Time Keywords

Date and Time keywords describe the file creation and observation events. All dates are in the yyyy-mm-dd notation, e.g., ‘2004-06-15’. All times are in the sexagesimal notation (hh:mm:ss.ss), e.g., ’09:10:23.85’.

a) LOIS Standard

DATE	String	/	UT date and time of file creation (file creation is assumed to occur at the

beginning of the exposure), e.g., ‘2004-06-15T03:33:46.00’

DATE-OBS	String	/	UT date and time of exposure start, e.g., ‘2004-06-15T03:33:46.16’

UTCSTART	String	/	UTC of exposure start, e.g., ’09:10:23.00’

UTCEND	String	/	UTC of exposure end, e.g., ’09:10:29.75’. This is the end of the exposure

for a single frame and the end of the last exposure for a time series.

LST-OBS	String	/	Local Sidereal Time of exposure start, e.g., ’18:25:15.00’, from HK key

coord.lst

EXPTIME	Float	/	Exposure time in seconds. (See discussion of time at the end of section 1)

TIMESRC	String	/	Indicates the manner in which the time was set. It is ‘GPS-QMM’,

‘NTP’, ‘SYSTEM CLOCK’, ‘UNKNOWN’.

TIMEQUAL	String	/	If TIMESRC=GPS-QMM this is the Trak 8821 RQIR response string,

usually, but not always, of the form ‘INT REF-UTC XXX ns’.

If TIMESRC=NTP the content of this keyword is TBD.

For other TIMESRC cases this string will be ‘UNKNOWN’.

GPSSTATE	String	/	If TIMESRC=GPS-QMM this is the Trak 8821 RQLK response string,

‘LOCKED SINCE ddd:hh:mm:ss’, ‘FREE RUN SINCE ddd:hh:mm:ss’, or

‘NO LOCK SINCE POWERUP’.

For other TIMESRC cases this string will be a blank string, ‘ ’.

GPSVERS	String	/	GPS module software version.

b) SOFIA Required

NONE

c) SOFIA Optional

NONE

## Environmental Keywords

Environmental keywords describe the environment during observation.

a) LOIS Standard Keywords

NONE

b) SOFIA Required

WVZ_STA	Float	/	Water vapor column density in microns Hg at UTCSTART,

from HK key wvm_if.wvmdata.water_vapor

WVZ_END	Float	/	Water vapor column density in microns Hg at UTCEND, 						from HK key wvm_if.wvmdata.water_vapor

TEMP_OUT	Float	/	Outside static air temperature in degrees Celsius,

from HK key das.ic1080_2hz.static_air_temp

TEMPPRI1	Float	/	Primary mirror temperature #1 in degrees Celsius,

from HK key ta_mcp.mcp_hk_pms.pms_temp_1

TEMPPRI2	Float	/	Primary mirror temperature #2 in degrees Celsius,

from HK key ta_mcp.mcp_hk_pms.pms_temp_2

TEMPPRI3	Float	/	Primary mirror temperature #3 in degrees Celsius,

from HK key ta_mcp.mcp_hk_pms.pms_temp_3

TEMPSEC1	Float	/	Secondary mirror temperature #1 in degrees Celsius,

from HK key a_mcp.mcp_hk_pms.sm_temp_1

AIRSPEED	Float	/	True aircraft airspeed in knots, from HK key

das.ic1080_10hz.true_airspeed

GRDSPEED	Float	/	Aircraft ground speed in knots, from HK key

das.ic1080_2hz.ground_speed

HEADING	Float	/	Aircraft heading in degrees, measured from North through East.

from HK key das.ic1080_2hz.true_heading

LON_STA	Float	/	Aircraft longitude in degrees at UTCSTART, measured from the

Prime Meridian with positive values going East, from HK key

das.ic1080_2hz.lon_fms_1. The values of this keyword and the next five

keywords come from the aircraft control system, as opposed to the

keywords HGPS_LON, HGPS_LAT and HGPS_ALT in (3.a), which come

from the HIPO GPS unit for the corresponding channel. Please note that

HGPS_ALT in (3.a) is in meters (1 foot equals 0.305 meters).

LON_END	Float	/	Aircraft longitude in degrees at UTCEND,

from HK key das.ic1080_2hz.lon_fms_1

LAT_STA	Float	/	Aircraft latitude in degrees at UTCSTART,

from HK key das.ic1080_2hz.lat_fms_1

LAT_END	Float	/	Aircraft latitude in degrees at UTCEND,

from HK key das.ic1080_2hz.lat_fms_1

ALTI_STA	Float	/	Aircraft pressure altitude in feet at UTCSTART,

from HK key das.ic1080_15hz.press_alt

ALTI_END	Float	/	Aircraft latitude in degrees at UTCEND,

from HK key das.ic1080_15hz.press_alt

TRACKANG	Float	/	Aircraft track angle, from HK key das.ic1080_2hz.true_track_angle

c) SOFIA Optional

NONE

## Camera Keywords

Camera keywords describe the detector, detector mode, and detector environment.

LOIS operates many different CCD camera systems, so the camera keywords must be capable of describing the layout for all the detectors it operates. In this document we describe a dual-amplifier CCD as shown in Figure 2. There are two regions in two columns, corresponding to the two readout amplifiers. The regions (and amplifiers) are numbered 1 to 2 left to right. The coordinate system is (X,Y) across the whole CCD (i.e., there is no subdivision by amplifiers) and follows the usual convention (X increasing left to right, Y increasing bottom to top). Postscan, prescan and overclock pixels are not part of the overall coordinate system. After the readout the raw data from the two amplifiers are de-interlaced and so the final FITS file will have a different layout, with the prescan, postscan, and overclock pixels placed outside of the image as explained later and shown in Figures 1 and 2.

The effect of binning on the prescan, postscan and overclock pixels:

The prescan pixels are in a fixed number of physical pixel columns that result from additional pixels in the CCD serial registers between the on-chip amplifiers and the first column in the CCD imaging array. These pixels do not see light, nor have dark current. The postscan pixels, on the other hand, are non-physical pixels used for bias determination. It is usually desirable to have a fixed number of them regardless of how the CCD is binned for readout. The result is that if P is the number of prescan columns and B is the binning factor, there will be P/B data elements of prescan. If P/B is not an integer, the first real pixel in the imaging area will be partially binned with prescan pixels. On the other hand, there always will be the same (fixed) number of postscan data elements regardless of the binning factor. It is possible, depending on the binning factor and image size, for the last pixel in an image row to include some amount of overscan through the summing register. The overclocked rows act just like the postscan columns, i.e. there will be some fixed number of them in the readout data regardless of binning. There is no such thing as prescan rows.

a) LOIS Standard

DETECTOR	String	/	Camera detector name.

CAMVERS	String	/	LOIS camera module software version.

CAMMODE	String	/	Camera exposure mode. Currently defined modes are Single Frames,

Fast Dots, Slow Dots, Strips, Find, Series, Basic Occultation,

Fast Occultation, Pipelined Occultation.

CCDCFGNM	String	/	The name of the CCD camera configuration file in /opt/LOIS/config, which

includes CCD geometry, DSP code parameters, temperature calibrations,

and other quantities related to the operation of the CCD.

TRIGGER	String	/	This keyword may be either ‘HARDWARE’ or ‘SOFTWARE’, depending on

which type of triggering was used.

INTERLCE	Boolean	/	If the value is T (True), it means that the FITS file contains the raw

(interlaced) data stream from the amplifiers. The user will have to

rearrange the data so that the output from each amplifier is placed in its

proper quadrant in the proper order in the final image.

DETSIZE	String	/	Detector size in pixels (e.g., ‘2096 x 1036’).

PIXSIZE	Float	/	Pixel size in microns. It is fixed by the CCD design and is not affected by

binning.

PIXSCAL	Float	/	Unbinned pixel scale in arcseconds per pixel. Not affected by binning.

DETTEMP	Float 	/	Detector temperature in degrees Celsius.

SETTEMP	Float 	/	Detector temperature set value in degrees Celsius.

CTTEMP	Float	/	Controller temperature in degrees Celsius.

PIXTIME	Float	/	Readout time in microseconds per data element. Its value depends on the

binning factor in a linear, but not proportional manner up to binning by

five. For binning factors of 6 or more this value is set to zero and the pixel

time needs to be measured.

INT_TIM	Float	/	Integration time in microseconds of one ramp of the dual-slope integrator.

R_DELAY	Float	/	Serial clock overlap time in microseconds.

SI_DELAY	Float	/	Parallel clock overlap time in microseconds.

TRIGCNT	Integer	/	Actual number of CCD triggers reported at the end of an observation. This

is always somewhat more than TRIGNOM, the difference increasing with

the trigger rate. A discrepancy of a factor of two or more indicates that the

trigger rate is too high for the readout time and triggers are being missed.

Value is zero if software triggered.

TRIGNOM	Integer	/	Calculated number of CCD triggers in the observation, normally equal to

NAXIS3 + 1. Value is zero if software triggered.

SUBARRNO	Integer	/	Number of subarrays in observation. Zero for full frame, n for n subframes

SUBARSER	Integer	/	Number of the subarray in this file. Zero for full frame.

NUMAMP	Integer	/	Number of readout amplifiers used in the observation.

PRESCAN	Integer	/	The number of prescan columns in data elements. This number is not

amplifier-specific and is equal to the number of prescan column pixels

(fixed by the CCD design, 8 for HIPO) divided by the binning factor in

the column direction, CRDELT1. At present (August, 2008) HIPO

subframes do not contain prescan pixels and full width frames do.

POSTSCAN	Integer	/	Number of postscan columns in data elements. This number is the same

for all amplifiers and is not affected by binning.

POSTCLK	Integer	/	Number of overclocked rows in data elements. This number is the same

for all amplifiers and is not affected by binning. Overclock is currently not

implemented and POSTCLK is always zero.

The next ten keywords describe the WCS system which maps the data elements to the sky.

CTYPE1	String	/	Name of the coordinate represented by WCS axis 1 (e.g., ‘RA – TAN’).

CTYPE2	String	/	Name of the coordinate represented by WCS axis 2 (e.g., ‘DEC – TAN’).

CRPIX1	Float	/	The location of a reference point along WCS axis 1, in units of the data

index along NAXIS1.

CRPIX2	Float	/	The location of a reference point along WCS axis 2, in units of the data

index along NAXIS2.

CRVAL1	Float	/	The value of the coordinate CTYPE1 at reference point CRPIX1 (e.g., RA

in degrees).

CRVAL2	Float	/	The value of the coordinate CTYPE2 at reference point CRPIX2 (e.g., Dec

in degrees).

CDn_m	Float	/	The WCS matrix coefficient relating the data index along axis n to the

Sky coordinates along axis m.  The values for n and m run from

1 to the value of the NAXIS keyword.

The next nine keywords (AAMP_##, AGAIN_##, ARDNS_##, AORGX_##, AORGY_##, AENDX_##, AENDY_##, ADELX_##, and ADELY_##), or A-keywords, are amplifier-specific keywords. They are used to describe the size, location and physical properties of the region of the CCD read out by each amplifier. All AORG and AEND keyword values are in unbinned pixels and are set to -1 if the corres-ponding amplifier is not in use. As with CRVAL1U and CRVAL2U these will be integer multiples of the binning factor. The AORG, AEND, and ADEL keywords are used to describe the location and binning (if any) of the subarray(s) when only a portion of the CCD is being read. The AAMP keywords can have values of 0 (zero) or 1 (one) only; they show which amplifiers have been used in acquiring the image and are useful for simplifying the calculations of the location and size of the prescan, postscan, overclock and image areas. Since the HIPO CCDs have only two amplifiers only the keywords for amplifiers 1 and 2 are present. At present subframes can only be used with a single amplifier.

AAMP_##	Integer	/	1 if amplifier ## has been used, 0 otherwise

AGAIN_##	Float 	/	Gain of the array read by amplifier ## in electrons per ADU.

ARDNS_##	Float 	/	Readout noise of the array read by amplifier ## in electrons.

AORGX_##	Integer	/	X position of first silicon pixel read by amplifier ## (in unbinned pixels).

AORGY_##	Integer	/	Y position of first silicon pixel read by amplifier ## (in unbinned pixels).

AENDX_##	Integer	/	X position of last silicon pixel read by amplifier ## (in unbinned pixels).

AENDY_##	Integer	/	Y position of last silicon pixel read by amplifier ## (in unbinned pixels).

ADELX_##	Integer	/	Binning factor for amplifier ## in the X-direction. The value of this keyword

is positive for amplifier 1 and negative for amplifiers 2.

ADELY_##	Integer	/	Binning factor for amplifier ## in the Y-direction. The value of this keyword

is positive for amplifier 1 and negative for amplifier 2.

When the entire CCD is being read, each amplifier reads its half of the CCD. When only portions of the CCD are read, the subarray(s) cannot cross amplifier boundaries. By default the FITS file contains de-interlaced data (keyword INTERLCE is equal to F) so that the portions read by each amplifier are assembled together to form a seamless image.

The prescan columns are put to the left (at low column numbers) and the postscan columns are put to the right (at high column numbers) of the de-interlaced image. This arrangement results in a seamless image with all auxiliary rows and columns (prescan, postscan and postclock) outside of the useful area of the image, as shown in Figures 1 and 2. The overclock rows are placed at the top of the image (at high row numbers).

The location and order of the prescan and postscan regions is the same as that of the corresponding amplifiers. For example, the postscan pixels for amplifier 1 will be to the left of those for amplifier 2 (see Figures 2). The height of the prescan/postscan regions can be found from the AORGY_## and AENDY_## keywords. If both amplifiers are used, the final image will have two overclock regions at the top, each of height POSTCLK (see Figure 2)

b) SOFIA Required

SUBARR##	String	/	This keyword appears in the header only if there are subrames. For each

subframe in this observation the string is in the IRAF style notation 				[x1:x2,y1:y2], e.g. [213:445,128:275]. The string is

[AORGX_xx:AENDX_xx,AORGY_xx:AENDY_xx] for amplifier xx

and subframe ##. SUBARSER is the subframe whose data are in this file.

c) SOFIA Optional

NONE

## Telescope Keywords

Telescope keywords describe the telescope, position, and configuration.

a) LOIS Standard

TELESCOP 	String	/	Telescope name

TELVERS	String	/	LOIS telescope module software version

TELRA 	String	/	Right Ascension (hh:mm:ss) as returned from TCS, from HK key

coord.pos.sibs.ra

TELDEC 	String 	/	Declination (dd:mm:ss) as returned from TCS, from HK key

coord.pos.sibs.dec

OBSRA	String	/	Requested RA value (hh:mm:ss), from HK key coord.pos.sibs.ra

OBSDEC	String	/	Requested DEC (dd:mm:ss), from HK key coord.pos.sibs.dec

EQUINOX	Float	/	Equinox of RA and DEC.

ZA	Float	/	Zenith angle in degrees, from HK key coord.pos.sibs.alt

HA	String	/	Hour angle (hh:mm:ss).

AIRMASS 	Float	/	Airmass, secant of ZA, from HK key coord.pos.sibs.alt

TELFOCUS	Float	/	Telescope focus position, from HK key ta_scs.fcm_status.fcm_act_t

MCSTIME	Float	/	Time reported on  the MCS

b) SOFIA Required

ZA_START	Float	/	Zenith angle (in degrees) at start of exposure,

from HK key coord.pos.sibs.alt

ZA_END	Float	/	Zenith angle (in degrees) at end of exposure,

from HK key coord.pos.sibs.alt

FOCUS_ST	Float	/	Telescope focus position at UTCSTART, from HK key

ta_scs.fcm_status.fcm_act_t

FOCUS_EN	Float	/	Telescope focus position at UTCEND, from HK key

ta_scs.fcm_status.fcm_act_t

TSC-STAT	String	/	TASCU status as reported by the HK key ta_tsc.tsc_mcs_hk.tsc_status.

For example, 'STAB_INERTIAL_ONGOING'

FBC-STAT	String	/	FBC status at start of observation as reported by the HK key

ta_tsc.tsc_mcs_hk.fbc_status (e.g., ‘FBC_ON’).

AOI-ID	Integer	/	Current AOI in use by the TA tracker system, in the range of 1-9,

from HK key ta_trc.trc_tracking_table.cent_ref_track_aoi

SIBS_X	Float	/	X-coordinate of SIBS from MCCS, from HK key

coord.pos.sibs.xsi

SIBS_Y	Float	/	Y-coordinate of SIBS from MCCS, from HK key

coord.pos.sibs.ysi

TELEQUI	String	/	Equinox of MCCS ERF coordinates, from HK key coord.pos.sibs.equinox,

e.g. ‘J2000.0’

TELCONF	String	/	Telescope configuration. It has the fixed value of ‘NASMYTH” for SOFIA.

TRACMODE	String	/	SOFIA tracking mode: OnAxis or Offset, as reported by the HK key

ta_trc.trc_status_table.main_op_mode_id

TRACERR	Boolean	/	Error in tracker during observation.

TELVPA	Float	/	Position angle of up on the CCD, measured in degrees S −> W,

from 0 to 360, as reported by HK key coord.pos.sibs.vpa

TELLOS	Float	/	Telescope LOS at UTCSTART, from HK key coord.pos.sibs.los

TELXEL	Float	/	Telescope cross-elevation at UTCSTART, from HK key coord.pos.sibs.xel

TELEL	Float	/	Telescope elevation at UTCSTART, from HK key coord.pos.sibs.el

LASTREW	String	/	The time since the last LOS rewind (currently unavailable from the MCCS).

MCCSMODE	String	/	Current MCCS instrument observing mode for HIPO or FLIPO, as reported

by the HK keys flipo.si_config.current_mode or

hipo.si_config.current_mode (e.g., ‘FLITECAM_IMAGING’).

c) SOFIA Optional

TELTCS	String	/	Name of MCS software and version, e.g. MCS V1.0

## Annotation Keywords

This section describes the annotation keywords, only COMMENT for HIPO.

a) LOIS Standard

COMMENT	String	/	Standard FITS text comment, no equal sign.

b) SOFIA Required

NONE

c) SOFIA Optional

NONE

## Instrument Keywords

Instrument keywords describe the instrument and instrument configuration. Keyword values are padded with spaces if necessary to make a minimum of 8 characters between the single quotes.

a) LOIS Standard

INSTRUME	String	/	Instrument name, either HIPO RED or HIPO BLUE

INSTMODE	String	/	Instrument observing mode:

'Single '

'Slow Dots'

'Fast Dots'

'Find '

'Strips '

'Basic Occultation'

'Fast Occultation'

'Pipeline Occultation'

DATATYPE	String	/	Data type (image, spectrum, etc.)

INSTCFG	String	/	Instrument configuration: IMG-RED, IMG-BLUE or IMG-DUAL

INSTLCFG	String	/	Installation configuration number from drawing HI-P00-00

ASMBCFG	Integer	/	Assembly configuration number (1-6) from drawing HI-P01-00

INSTVERS	String	/	LOIS instrument module software version.

INSTFOC	Integer	/	Focus Position of Camera Lens

INSTWHL	Integer	/	Number of instrument wheels. 1 for HIPO.

FILT_1	Integer	/	Filter position for wheel #1. Each side of HIPO has only one wheel.

FILTNME1	String	/	Filter name for wheel #1 (e.g., B,V,R,I or custom name).

WAVECENT	Float	/	Central wavelength in microns based on the filter in use.

BANDWDTH	String	/	Wavelengths in microns of 50% transmission points of bandpasses in

the form [min,max] (eg. [0.49,0.59]. The Johnson filter cases factor in

the CCD QE curve and assume no clipping by the dichroic. The open

cases factor in the dichroics and CCD but ignore the FLITECAM dichroic.

PUPIL	Boolean	/	True for Pupil Viewing; False for Direct Imaging

SHK-HART	Boolean	/	True for Shack-Hartmann beamsplitter in the field

SH_LED	Float	/	LED voltage setting for the Shack-Hartmann LED, ADUs

RR_LED	Float	/	LED voltage setting for the retroreflection LED, ADUs

For both of these keywords, values of 2048 or less turn off the LED and
			4095 is as bright as possible.

CHOPTRG	Boolean	/	True if square wave chopper trigger signal is running

CHOPHZ	Float	/	Frequency of square wave chopper trigger

b) SOFIA Required

SPECTEL1	String	/	Spectral element #1 (same as FILTNME1). All possible values:

''HIP_U’

''HIP_B'

''HIP_V'

''HIP_R'

''HIP_I'

HIP_SDSS-U

HIP_SDSS-G

HIP_SDSS-R

HIP_SDSS-I

HIP_SDSS-Z

'SH497_97_H'

'SH768_86_S'

'SH1750_90_S'

‘SH1060_260_S ‘ (TBC)

'HIP_F9500'

'HIP_METH_890'

‘BLANK’

OPEN

'CUSTOM'

SPECTEL2	String	/	Spectral element #2. Always ‘NONE ‘.

c) SOFIA Optional

MCSTIME	String	/	UNIX time at start, as reported by the HK key , e.g. ‘1366962321.42’

## Data Collection Keywords

These keywords describe the data collection environment.

a) LOIS Standard

NONE

b) SOFIA Required

CHOPPING	Boolean	/	Chopping flag.

NODDING	Boolean	/	Nodding flag.

DITHER	Boolean	/	Dithering flag.

MAPPING	Boolean	/	Mapping flag.

SCANNING	Boolean	/	Scanning flag.

c) SOFIA Optional

NONE

## DCS Pipeline Related Keywords

These keywords describe the current state of the Data Cycle System’s pipelined analysis. HIPO doesn’t use this so the data archived by the DCS is all LEVEL_1.

a) LOIS Standard

NONE

b) SOFIA Required

PROCSTAT	String	/	Processing status. Set to LEVEL_1 for HIPO.

HEADSTAT	String	/	Header status (e.g., ‘ORIGINAL’), as written by LOIS.

c) SOFIA Optional

NONE

## Mission Management Keywords

These keywords describe the Mission Management Support information.

a) LOIS Standard

NONE

b) SOFIA Required

DEPLOY	String	/	Site deployment (e.g., DAOF).

FLIGHTLG	Integer	/	Flight leg.

MISSN-ID	String	/	Mission ID as returned by the MCCS, HK key mission_id.

c) SOFIA Optional

NONE

## Chopping Keywords

The Chopping keywords describe the chopper parameters. At the time of release of this document none of these keywords appear in the header because CHOPPING=F.

a) LOIS Standard

NONE

b) SOFIA Required (if chopping)

None used by HIPO.

c) SOFIA Optional

NONE

## Nodding Keywords

These keywords describe the nodding parameters. At the time of release of this document none of these keywords appear in the header because NODDING=F.

a) LOIS Standard

NONE

b) SOFIA Required (if nodding)

None used by HIPO.

c) SOFIA Optional

NONE

## Dithering Keywords

These keywords describe the dithering parameters. At the time of release of this document none of these keywords appear in the header because DITHER=F.

a) LOIS Standard

NONE

b) SOFIA Required (if dithering)

None used by HIPO.

c) SOFIA Optional

None used by HIPO.

## Mapping Keywords

These keywords describe the Mapping/Scanning parameters. At the time of release of this document none of these keywords appear in the header because MAPPING=F.

a) LOIS Standard

NONE

b) SOFIA Required (if mapping)

None used by HIPO.

c) SOFIA Optional

None used by HIPO.

## Scanning Keywords

These keywords describe the Mapping/Scanning parameters. At the time of release of this document none of these keywords appear in the header because SCANNING =F.

a) LOIS Standard

NONE

b) SOFIA Required (if scanning)

None used by HIPO.

c) SOFIA Optional

NONE

# HIPO Camera Modes

The HIPO CCDs (e2v CCD47-20) are dual-amplifier frame transfer detectors (see Figure 5). They have a 1024x1024 pixel active imaging area (1056x1027 including masked pixels for dark reference) and a second 1056x1033 area covered by aluminum that is called the storage area. It is possible to clock the integrated charge image from the image area to the storage area using HIPO’s AIMO CCDs in 7-12 ms depending on the saturation behavior required. It may be possible to speed up the frame transfer, and NIMO versions of the CCD47-20 can complete the frame transfer in as little as 2 ms. The CCD47-20 has one serial register located at the bottom edge of the storage area. There is an output amplifier at each end of the serial register so the CCD can be read entirely through one or the other amplifier or it can be split so one half is read through one amplifier while the other is simultaneously read through the other amplifier. The charge image in the storage area can be read out while the image area is integrating for the next readout. This overlapping of activities at the detector level greatly reduces the dead time in time-resolved imaging applications.

Find mode is the only mode that cannot make use of hardware triggering. All other modes are capable of hardware triggering, though an alternative operation with software triggering is also possible. The timing accuracy is of course much poorer with software triggering. This is included largely as a fallback operation mode in case of unrecoverable problems with the hardware triggering system. The TRIGGER keyword indicates which type of triggering was used.

All modes can make use of pixel binning with arbitrary binning factors. The DSP code provided by ARC can handle binning in a very general manner, but the loop and subroutine call overhead is unacceptably high for the demands of HIPO. We have augmented this binning code with in-line serial read code for serial binning factors up to 5. Larger binning factors use the general code from ARC with its associated timing penalty.

The manner in which the sub-frames (also called subarrays in Section I) are documented in the header is described earlier. The text "read out specified CCD region" in the following descriptions is intended to mean that LOIS carries out the process needed to readout whatever sub-frame(s) have been defined for readout. There is an additional level of detail in this area that is not described here that has bearing on the frame rate capability of the instrument. Examples include binning factor selection, number of sub-fames together with their size and position, and number of amplifiers used during readout. These all affect the performance of the instrument but not the organization of the data file header or instrument mode definitions. In the same vein there is no discussion of image display or analysis since these are not data interface issues. At present (August 2008) subframes have a number of restrictions: 1) they can only be used with a single amplifier readout; 2) if more than one subframe is used during a readout, all subframes have the same size; 3) the maximum number of subframes read out simultaneously is nine, and 4) multiple subframes must not overlap in the row direction. The various modes deal with subframes in different ways. Please see the mode descriptions in the following subsections.

The table below lists the currently defined HIPO camera modes and their associated header keywords.

Mode	NAXIS	NAXIS2	NAXIS3	CD3_3U	EXPTIME

Find	2	# rows	NA	NA	Integration length

Single Frames	2	# rows	NA	NA	Shutter open time

Series*	3	rows/frame	# Frames	sec/frame	Shutter open time

Basic Occultation	3	rows/frame	# Frames	sec/frame	Integration length

Fast Occultation	3	rows/frame	# Frames	sec/frame	Integration length

Pipelined Occ’n	3	rows/frame	# Frames	sec/frame	Integration length

Fast Dots	3	rows/dot	# dots	sec/dot	Integration length

Slow Dots	3	rows/dot	# dots	sec/dot	Shutter open time

Strips	3	1	# "rows"	sec/row	Integration length

* Series mode has not been implemented at present; basic occultation is a superior approach in most cases.

In the table the keyword EXPTIME represents the amount of time per frame that the CCD is static to integrate on the signal of interest. UTCSTART is the time of the beginning of the process for whatever mode is being used and UTCEND is the end of the whole process, such as a whole series of frames in a 3-D file. The difference between UTCEND and UTCSTART would be equal to EXPTIME in the NAXIS = 2 cases unless there is some sort of dead time involved. In general EXPTIME will be shorter than CD3_3U because of the dead time due to readout or parallel shifting depending on the mode. Most of the C*3 keywords are not included in the table because they are mode-independent if present. CRPIX3U will be 1, CRVAL3U is EXPTIME/2 for software triggering and CD3_3U–(EXPTIME/2) for hardware triggering, and CFINT3 will be the ratio EXPTIME/CD3_3U, the duty cycle of the mode. Please see the discussion of triggering at the end of Section I.1.

A brief description of the HIPO camera modes follows. Each description includes a pseudo-code definition of the mode. It is important to note that the files for some of the time-resolved modes can become very large. The current limit of ~2Gbytes is due to a 32-bit file length in Solaris, but this is likely to be increased to a 64-bit file length. In this event the maximum file size will be limited by available observing time and the maximum camera data rate of ~15 GB/hour. We understand that the DCS does not have problems with files up to 4 GB, possibly larger, at this time.

Description of the HIPO camera modes:

## Find

In Find mode the shutter is opened and the CCD read out repeatedly until interrupted by the observer. This mode is intended as an aid in field acquisition. Generally images obtained in find mode are not stored, although the last image prior to interruption may be manually saved if desired. A string of find mode images is started asynchronously. Images are read out with the desired integration interval unless the requested interval is too short to be possible. In this case images are obtained as rapidly as possible. This mode never uses hardware triggering. Full subframe capability is available in find mode.

An image manually saved in find mode is stored as a single 2-D FITS file with optionally included prescan, postscan, and overclocked pixels. If saved, the file index number is incremented.

Pseudo-code for Find mode:

build most of header

open shutter

stop CCD flush

do forever

build time-critical part of header (UTCSTART mainly)

wait till time to read out

if time is past, break out of wait loop

if abort, break out of wait loop and infinite loop, correct UTCEND, and set OBSSTAT

do frame transfer shift

read out specified CCD region (storage area) integrate (image area)

if abort

break out of loop

end do

close shutter

flush CCD continuously

manual save is an optional, independent process.

## Single Frames

This is a simple imaging mode. In its simplest form it takes a picture and stores it to disk, incrementing the file index number afterwards. It is also possible to request that several frames be taken with one command. These frames are not synchronized in any way – it is intended to be a shorthand way to allow multiple images to be taken. Each image is in a separate sequentially numbered file just as if a single frame were to be taken several times. Full subframe capability is available in single frames mode.

The images are straightforward 2-D FITS files with optionally included prescan, postscan, and overclocked pixels as described earlier.

Pseudo-code for Single Frames mode:

foreach image

build most of header

clear CCD

build time-critical part of header (UTCSTART mainly)

open shutter starting on exact second

integrate for EXP starting on same exact second (hardware trigger)

if abort

break out of exposure, correct UTCEND, and set OBSSTAT

close shutter

do frame transfer shift

read out specified CCD region

if storing data

write 2D FITS file for this image

increment file number

if abort

break out of loop

next image

flush CCD continuously

## Series

Series mode has not been implemented at this time because basic occultation mode is ordinarily a superior observing approach with a frame-transfer CCD. This section describes the operation of series mode were it to be implemented.

In Series mode a hardware timed, evenly spaced set of shuttered images is obtained. The exposure time of each image is defined by the mechanical shutter. For exposure times equal to or longer than the readout time the basic occultation mode (following section) is preferred because the frame transfer can occur more quickly than the mechanical shutter can respond. This mode will usually be used only if the exposure time must be shorter than the time required to read out the storage area. Full subframe capability is available in single frames mode.

The images are 3-D FITS files organized as a stack of 2-D images. The third axis is time. Each 2-D image in the 3-D file is stored with optionally included prescan, postscan, and overclocked pixels. These files can become very large. The duty cycle is relatively poor because the dead time per image is the readout time.

Pseudo-code for Series mode:

build most of header

build time-critical part of header (UTCSTART mainly)

continue flushing CCD until hardware trigger

foreach image

open shutter based on hardware trigger; first is on exact second

integrate for EXP based on hardware trigger; first is on exact second

if abort

break out of exposure

close shutter

do frame transfer shift

read out specified CCD region (storage area); integrate (image area)

if storing data

append to 3D FITS file

if abort

close shutter

break out of loop

correct UTCEND and set OBSSTAT in FITS header

next image

flush CCD continuously

if storing data

increment file number

## Basic Occultation

This mode is the most flexible high-speed time resolved imaging mode for HIPO. This mode is rigorously timed by using the hardware triggering electronics. The mechanical shutter remains open during the entire time series observation with the frame transfers defining the integration boundaries. This mode is distinguished by the fact that the full 1033 row frame transfer is done at the completion of each integration. There is no preferred location for the object of interest. The maximum frame rate depends on readout details. Full subframe capability is available in single frames mode.

The images are 3-D FITS files organized as a stack of 2-D images. The third axis is time. Each 2-D image in the 3-D file is stored with optionally included prescan, postscan, and overclocked pixels. These files can become very large.

Pseudo-code for Basic Occultation mode:

build most of header

build time-critical part of header (UTCSTART mainly)

continue flushing CCD until hardware trigger

open shutter starting on exact second

foreach image

integrate for EXP based on hardware trigger; first is on exact second

if abort

break out of exposure

do frame transfer shift (1033 rows)

read out specified CCD region (storage area); integrate (image area)

if storing data

append to 3D FITS file

if abort

break out of loop

correct UTCEND and set OBSSTAT in FITS header

next image

close shutter

flush CCD continuously

if storing data

increment file number

## Fast Occultation

This mode is a more efficient but more specialized variation on the basic occultation mode. It is rigorously timed by using the hardware triggering electronics. The mechanical shutter remains open during the entire time series observation with partial frame transfers defining the integration boundaries. This mode is distinguished by the fact that only a partial frame transfer (of NAXIS2 rows) is done at the completion of each integration. At this time the image area begins integrating and the charge image from the previous integration is shifted the rest of the way down to the serial register as the next integration is proceeding. This reduces the dead time due to parallel shifting compared to the basic occultation mode. There is a preferred location for the object of interest, namely in the first NAXIS2xCRDELT2 rows above the boundary between the image and storage areas. This mode supports only one subframe, and indeed, requires one subframe. The maximum frame rate depends on readout details.

It is helpful to consider an example. Assume we have a sub-frame with NAXIS2 of 100 unbinned pixels. In basic occultation mode the dead time from the frame transfer would be 7-12 ms, but in fast occultation mode it would be only 0.7-1.2 ms, 10 times less. The difference is that the remaining 933 rows of parallel clocking would be overlapped with the next integration in the image area. The integration interval would be the same as for basic occultation mode because the full 1033 rows of parallel shift still occur for each integration.

Because only a partial frame transfer occurs, each image is exposed to sky for a longer period than the integration time on the object of interest. Because of the high speeds involved (i.e., if the 7-12 ms dead time of basic occultation mode is of concern, the integration interval must be on the order of 100 ms or less). This is not a serious issue and is an acceptable trade-off. If necessary a focal plane mask can be introduced to reduce this problem.

The images are 3-D FITS files organized as a stack of 2-D images. The third axis is time. Each 2-D image in the 3-D file is stored with optionally included prescan and postscan pixels. These files can become very large.

Pseudo-code for Fast Occultation mode:

build most of header

build time-critical part of header (UTCSTART mainly)

continue flushing CCD until hardware trigger

open shutter starting on exact second

foreach image

integrate for EXP based on hardware trigger; first is on exact second

if abort

break out of exposure

do partial frame transfer shift (NAXIS2xCRDELT2 rows)

do charge image shift (1033 - NAXIS2xCRDELT2 rows) (storage area); integrate (image area)

read out specified CCD region (storage area); integrate (image area)

if storing data

append to 3D FITS file

if abort

break out of loop

correct UTCEND and set OBSSTAT in FITS header

next image

close shutter

flush CCD continuously

if storing data

increment file number

## Pipelined Occultation

Pipelined occultation mode is a modification of fast occultation mode that is faster but even more specialized. It uses the same DSP code as fast occultation mode with the exception that the final shift of the image down the frame store region to the serial register prior to readout does not occur. In this case the partial frame transfer of fast occultation mode is followed directly by a read of the same number of rows. The difference between fast occultation and basic occultation is one of duty cycle; the shortest integration interval remains the same since the charge image is shifted the full height of the frame store area prior to readout. Pipelined occultation has the same duty cycle as fast occultation but reduces the shortest possible interval because the bulk of the frame transfer (taking approximately 7-12 ms) does not occur for each image.

Pipelined occultation mode is tricky to use. Ideally the height of the (single) subframe would be an odd integer submultiple of the height of the frame store area, but that is 1033 rows, a prime number! Fortunately being close is good enough if one is willing to accept a few bad rows at either the top or bottom of the subframe. As an example to see how this works, imagine an unbinned subframe 344 rows tall. Three such subframes fit in the storage area (with one additional row left over). An object within the first 344 rows above the seam between the image and frame store areas leaves a charge image that is shifted down into the top third of the frame store area on the first trigger. After this shift the bottom third of the frame store area is read out, a spurious frame at the beginning of the pipeline. When the read is finished the original charge image is in the middle third of the frame store area. On the next trigger the second charge image is moved to the top third of the frame store area and the first image is now in the bottom third. When the read happens, the second frame is not spurious, but is the first charge image. If there are 2n+1 subframe heights in the frame store area there will be n spurious pipeline frames before the first real image. If the number of subframes in the frame store area is even, all of the good images will be dumped during the partial frame transfer and all the reads will be empty.

Because only a partial frame transfer occurs, each image is exposed to sky for a longer period than the integration time on the object of interest. Because of the high speeds involved (i.e., if the 7-12 ms dead time of basic occultation mode is of concern, the integration interval must be on the order of 100 ms or less). This is not a serious issue and is an acceptable trade-off. If necessary a focal plane mask can be introduced to reduce this problem.

The images are 3-D FITS files organized as a stack of 2-D images. The third axis is time. Each 2-D image in the 3-D file is stored with optionally included prescan and postscan pixels. These files can become very large.

Pseudo-code for Pipelined Occultation mode:

build most of header

build time-critical part of header (UTCSTART mainly)

continue flushing CCD until hardware trigger

open shutter starting on exact second

foreach image

integrate for EXP based on hardware trigger; first is on exact second

if abort

break out of exposure

do partial frame transfer shift (NAXIS2xCRDELT2 rows)

read out specified CCD region (storage area); integrate (image area)

if storing data

append to 3D FITS file

if abort

break out of loop

correct UTCEND and set OBSSTAT in FITS header

next image

close shutter

flush CCD continuously

if storing data

increment file number

## Fast Dots

This mode allows acquisition of a finite string of images at very high frame rates using the CCD as an analog storage buffer. The star image should be placed on the CCD near the edge farthest from the serial register. The mechanical shutter is opened, and after every integration interval the parallel clocks are activated to shift the integrated charge image toward the serial register by the number of rows represented by NAXIS2xCRDELT2. When the requested number of shifts has been carried out the shutter is closed and the CCD is read out. The integration interval is limited only by the time required to clock the parallels by the number of rows requested. Fast dots uses a single subframe whose width can be the full width of the CCD; multiple subframes are not supported. The integration interval is not affected by the subframe width. The vertical position of the subframe is not used.

If the file is displayed as a 2-D image it appears to be a string of images arranged vertically and separated by the number of rows defined by NAXIS2xCRDELT2. This is in fact the form of the charge image in the CCD at the time readout is started. We have chosen to represent a fast dots dataset as a 3-D FITS file with the number of images being NAXIS3. If this file is viewed as a very short movie, it consists of NAXIS3 images of size NAXIS1xNAXIS2 data elements.

The star should be located about NAXIS2/2 rows from the edge of the CCD farthest from the serial register with NAXIS2 chosen so that 2060 (the number of rows in the combined image and storage areas) is divisible by NAXIS2. NAXIS3 would then be set to 2060/NAXIS2. This gives the maximum number of images in the file and also gives a deterministic behavior for timing. If the star image is located closer to the serial register, fewer images can be obtained. Timing will still be correct if NAXIS3 is reduced to correspond to the number of NAXIS2 shifts required to place the first charge image within NAXIS2 rows of the serial register upon completion of the dots data acquisition procedure.

It will often be the case for images obtained in this mode that the mechanical shutter's opening and closing times will be slow compared to the integration interval. This may cause the first images to be faint or entirely missing while the last image may be excessively bright. Also if the integration interval is not much larger than the required parallel shift time, the star images will appear to have streaks connecting them due to charge accumulation during the parallel shifts. As an example, consider a case with 500 μs integration interval with a parallel shift of 20 rows per image. The CCD is large enough to store 103 images so the total duration of the time series is 51.5 ms. The shutter opening and closing times may be 5 ms, so perhaps the first 10 images may be missing entirely. If the shutter is forced to stay open, this can be avoided. The parallel shift time may be, say, 7x20 = 140 μs, with a 360 μs static exposure time. The streaks joining the charge images will be pronounced in this case.

Pseudo-code for Fast Dots mode:

build most of header

build time-critical part of header (UTCSTART mainly)

continue flushing CCD until hardware trigger

open shutter starting on exact second

foreach image up to NAXIS3

integrate for EXPTIME based on hardware trigger; first is on exact second

if abort

break out of exposure

shift image down by NAXIS2xCRDELT2 value on hardware trigger signal

if abort

break out of loop

correct UTCEND and set OBSSTAT in FITS header

next image

close shutter

read out specified CCD region

if storing data

write to 3D FITS file

increment file number

flush CCD continuously

## Slow Dots

This mode produces data similar to the fast dots mode but instead of separating the individual images by parallel shifts only, the images are separated by full readout of the number of binned rows specified by NAXIS2. The result is that slow dots mode can produce an arbitrarily large number of images at the expense of a much longer shifting period than fast dots. In fast dots mode the shift period per image is NAXIS2xCRDELT2 times the parallel shift time, whereas in slow dots mode the shift time is NAXIS2 times the sum of the binned row read time and parallel shift time. Depending on the details of the readout this will be on the order of 100 times slower than for fast dots mode. Like fast dots, slow dots uses a subframe whose width can be the full width of the CCD, and multiple subframes are not supported. Unlike fast dots mode the integration interval does depend on subframe width. Because of the long shifting (readout) period each image is shuttered. It spite of its name, slow dots can take frames at a rate comparable to the speed of the shutter, so care should be taken that a life test of the shutter isn’t done by accident!

Slow dots is also similar to pipelined occultation mode except that the partial frame transfer is not done, only the read, so there are constraints on star position and NAXIS2 values that are similar to those for pipelined occultation mode. It is important to locate the star of interest approximately NAXIS2/2 rows above the dividing line between the storage and image areas so that the image is hidden behind the aluminum covering of the storage area after the read of NAXIS2 binned rows is complete. In order for the individual images of NAXIS2 binned rows to be read out of the CCD correctly, NAXIS2 is constrained by the requirement that 1033 be evenly divisible by NAXIS2xCRDELT2. As described in the pipelined occultation section 1033 is prime so this is impossible and the best that can be done is to choose NAXIS2xCRDELT2 so that it comes close to dividing 1033 evenly with a few rows short or a few left over.

The data produced by this mode is again a 3-D FITS file with NAXIS3 being the number of images obtained. If the star in question is located at row N (measured relative to the row in the storage area closest to the serial register), then the first N/(NAXIS2xCRDELT2) images will be blank. The next image in the file will be the image of the star at the integration beginning at UTCSTART. For example, if the star image is located 24 rows above the boundary between the image area and the storage area and NAXIS2 is 49, after 21 readouts the first star charge image would have been shifted toward the serial register by 21*49=1029 rows and the star image would be located 28 rows from the serial register. The 22nd readout would include the first star image centered at row 28. The effect of placing the star farther from the storage area boundary is to cause additional blank frames at the beginning of the slow dots series and a systematic offset in the timing of the series that must be accounted for by the observer.

Pseudo-code for slow dots mode:

build most of header

build time-critical part of header (UTCSTART mainly)

continue flushing CCD until hardware trigger

open shutter starting on exact second

foreach image

open shutter

integrate for EXPTIME based on hardware trigger; first is on exact second

if abort

break out of exposure

close shutter

read out NAXIS2 rows as specified on hardware trigger signal

if storing data

append to 3D FITS file

if abort

correct UTCEND and set OBSSTAT in FITS header

break out of loop

next image

flush CCD continuously

if storing data

increment file number

## Strips

This mode is exactly the slow dots mode with NAXIS2 = 1. It can be used in a variety of ways. We use it for ground-based astrometric imaging with the CCD columns oriented E-W, the telescope drive off, and the CCD clocked out at the same rate the sky drifts by. This creates an image that is arbitrarily long in the E-W direction and one CCD wide in the N-S direction. The same type of operation can be used on SOFIA to test telescope scanning with the scan direction parallel to the CCD columns. If the image is binned by a large factor in the parallel direction it is possible to obtain a high-speed one dimensional star position over long time periods. Finally, if a coarse grating or weak prism is incorporated in the optical system along with an aperture at the focal plane, a spectrally resolved time series can be obtained at high speed. This is useful for observing lunar occultations.

We list this as a separate mode because of the way it is used rather than the way it is implemented. It is exactly slow dots with NAXIS2 = 1 so no pseudo-code appears here.

# List of Searchable FITS Keywords

This is the preliminary list of FITS keywords that can be searched on. The list is in alphabetical order.

CAMMODE

DATE-OBS

OBJECT

OBJNAME

OBSDEC

OBSRA

OBSTYPE

UTCSTART

# Appendices

## FITS file naming convention

The name of each FITS file has a user-defined component and a system-generated component. The user supplies the first component, typically something like Red_20040615, meaning that the observation was taken on 15 June 2004 through the red channel. This part of the file name can also be any other string. The second part of the FITS file name consists of a prefix designating the subarray number if two or more subarrays are being read out. If only a single subarray or a full frame is taken the subarray section of the filename is not included. The filename ends with the sequential number of the observation, plus the extension “.fits”. The sequential observation number is 4 digits with leading zeroes. The subarray prefix is separated from the user-supplied string by the underscore character “_”. The observation number is separated from the subarray prefix (if present) or from the user-supplied string by a second underscore “_”. Separate data files are always generated for each subarray read out.

If an attempt is made to overwrite an existing data file LOIS prepends ‘a_’ to the filename and writes that file instead. If an a_* file already exists, LOIS prepends ‘b_’ instead, and so forth up to ‘z_’.

Examples:

Red_20040615_0005.fits: this is the fifth sequential observation, consisting of a single-frame

readout (there is no subarray prefix). Single-frame readout means either

the whole CCD or a single subarray.

Red_20040615_01_0912.fits: this is the 912th sequential observation, containing the

data for the first subarray in this observation. The presence of a subarray prefix

means that at least one more subarray has been read out. The subarray prefix is

the value of the SUBARSER keyword in the header.

Red_20040615_02_1012.fits: this is the 1012th sequential observation, containing the data for the

second subarray in this observation.

a_red_20040615_0003.fits: this filename is used if the file red_20040615.0003.fits would have

been overwritten.

-

-

## Items and questions still to be worked out

- Define TIMEQUAL when TIMESRC=NTP

# Figures

Figure 1: Layout of a single-amplifier CCD image

Q3

Q2

Postclock

\postscan for Q3

postscan for Q2

Overclock for Q2

Overclock for Q3

Postscan

Prescan

Postscan

Overclock for Q1

Overclock for Q4

Prescan for Q1

postscan for Q4

Prescan for Q4

postscan for Q1

A1

Q4

The small arrows in the bottom left corner show the direction of readout. Only the prescan pixels (which separate the amplifier from the clock lines) are physical pixels located on the CCD. They are additional pixels in the serial register and do not exist in the actual imaging area. NOTE: postclock (overclock) is currently not implemented. The POSTCLK keyword in the HIPO FITS headers will always have a value of 0.

Figure 2: Layout of a dual-amplifier CCD image

Postclock A1 AA1

Postclock A2 AA1

3

2

4

1

Q3

3

2

Q2

3

2

Postscan A1

Prescan A1

Postscan A2

Prescan A2

A2

A1

4

Q4

Q1

1

4

1

The small arrows in the bottom corners show the direction of readout. The prescans and postscans for the separate amplifiers are grouped on the left and right side of the image, respectively. NOTE: postclock (overclock) is currently not implemented. The POSTCLK keyword in the HIPO FITS headers will always have a value of 0.

1

2

3

4

2

3

1

4

Q2

2

3

4

1

Q1

Q3

Q4

1

2

3

4

2

3

1

4

Q2

2

3

4

1

Q1

Q3

Q4

Figure 3: The HIPO frame-transfer CCD

This figure is taken from the CCD47-20 datasheet and shows the layout of the HIPO CCDs in detail. These CCDs are intended to be operated at relatively warm temperatures where dark current is significant so there are 3 dark reference rows at the top and 16 dark reference pixels along both sides of the CCD. The active, light-sensitive image area is 1024x1024 pixels. The frame store area is the same width but 1033 pixels high. As noted in the readout mode pseudo-code sections, 1033 is a prime number and this complicates certain readout modes.

The serial register has independent clocks for the left and right halves. These can be ganged together to allow the entire CCD to be read from one amplifier or they can be operated separately to allow readout through both amplifiers. The 8 blank elements at each end of the serial register are the prescan pixels.